Approval Sheet Customer： Item： TOP VIEW LEDS Part No.: WLUW5050A-722-Y15 Customer P/N: 稳亮电子有限公司 WEN LIANG OPTO-ECTRONICS CO.LTD. Add: 4/F, Building 6,Tian Fu An Industrial Zone,Jiu weiVillage, xi xiang Town,bao an District shen zhen 518126,china Tel: 0086-755-29785289 Fax: 0086-755-29785189 E-mail: szwenliang@163.com Website: http://www.szwenliang.com.cn PREPARED BY; CHECKED BY; APPROVED BY; SIGNATURE; CHECKED BY; APPROVED BY. 供方签章： 客户签章： SUOOLIER： CUSTOMER： 日期： 日期： DATE： DATE： 备注：承认签章后请回复一份（或复印件）给我公司，其余由贵司留作存盘。如果在签章的承认书（或复印件）回复我司之前，下了有关此零件的订单且又无特殊说明，那么我司就确定贵司已完全承认。 Please return one specification or one copy of it with your chop and signature of approval and retain the others for your record.In the event of an order being placed for this part number before the chop and signed with specification (or copy) is returnde and without special explanation,it will be assumed that full approval have been given. # TYPE NO: WLUW5050A-722-Y15 # PACKAGE DIMENSIONS Note: 1. All Dimensions are in millimeters 2. Tolerance is ±0.1mm Unless otherwise specified. Device Selection Guide Emitting Color; Lens Type. White; ---. Absolute Maximum Ratings at Ta=25℃ Parameter; Symbol; Max; Unit. Power Dissipationhaosan\耗散功率; Pd; 0.2; W. Peak Forward Current 1/10 Duty Cycle,0.1ms Pulse Width 峰值正向电流; IFP; 150; mA. Forward Current正向电流; IF; 100; mA. Reverse Voltage反向电压; VR; 5; V. Operating Temperature Range 工作温度范围; Topr; -40℃ to+100℃. Storage Temperature Range 存储温度; Tstg; -40℃ to+110℃. Lead Soldering Temperature 【3mm From Body】焊接温度 [距车身3毫米]; Tsol; 260℃ For 5 Seconds. Electrical Optical Characteristics at Ta=25℃ Parameter; Symbol; Min; Typ; Max; Unit; Condition. Forward Voltage正向电压; VF; 2.8; 3.2; 3.5; V; IF=100mA. Luminous Intensity发光强度; IV; 40; 50; ----; Lm; IF=100mA. Color Rendering lndex 显色指数; Ra; ---; 80; 80; ---; IF=100mA. Chromaticity Coordinate色品坐标; X; 0.28; 0.30; 0.32; /; If=100 mA. Y; 0.29; 0.31; 0.33. Temperature Color色温; CCT; 6000; 6500; 7000; K; IF=100mA. Reverse Current反向电压; IR; /; /; 5; μA; VR=5V. Viewing Angle发光角度; 2θ1/2; /; 120; /; deg; IF=100mA. Note. 1.Tolerance of Luminous Intensity:11% 2.Tolerance of Dominant Wavelength:+1nm 3.Toleance of Forward Voltage:+0.1V Typical Electro-Optical Characteristics Curves Soldering condition Reflow Soldering; Hand Soldering. Lead Solder; Lead – free Solder. Pre-heat; 120~150℃; 180~200℃; Temperature; 350℃ Max.. Pre-heat time; 120sec. Max.; 120sec. Max.; Soldering time; 3sec. Max. (one time only). Peak temperature; 240℃ Max.; 260℃ Max.. Soldering time; 10sec. Max.; 10sec. Max.. Condition; refer to Temperature- Profile 1; refer to Temperature- Profile 2. Temperature profile in re-flow machine for lead type and lead free type Storage condition In order to avoid the absorption of moisture, it is recommended to store in the dry box (or desiccators) with a desiccant. Otherwise, to store them in the following environment is recommended. Temperature: 5℃~30℃ Humidity: 60%HR max. Dimension of carrier tape Dimension of reel Notes: 1. All dimensions are in mm, tolerance is±2.0mm unless otherwise noted. 2. Specifications are subject to change without notice. Packing 1. Number of SMD 5050 product in reel and bag(anti-static bag) 1.1. 1000 piece of SMD 5050 Products on one reel. The reel will be put in the anti-static bag then sealed. 1.2. 1000 piece of SMD 5050 products on one bag. 注意事项： - 通过使用适当的工具从材料侧面夹取，不可直接用手或尖锐金属压胶体表面，它可能会损坏内部电路 Through the use of appropriate tools from the material side of the clip, can not be directly used by hand or sharp metal pressure on the surface, it may damage the internal circuit - SMD吸咀外径不可超过LED尺寸，吸咀内役尺寸应尽可能大，吸咀顶端材质建议采用柔软材料以防在吸取期间刮伤或损坏LED胶体表面，元件的尺寸必须在取放机里准备的编程好，以确保精确的吸取和避免生产过中的损害 SMD suction nozzle diameter can not exceed the size of LED, the suction mouth battle size should be as large as possible, ceiling tip top material suggestions made of soft material to prevent absorb during scraping injury or damage the LED colloid surface, the size of the components must be in good pick and place machine to programming to ensure accurate absorption and avoid damage in production - 不可将模组材料堆积在一起，它可能会损坏内部电路 Do not stack module materials together, it may damage the internal circuit - LED工作环境与LED适配的材料中硫元素及化合物不可超过100PPM LED working environment is very suitable for the materials of LED and sulfur elements and compounds can not be more than 100PPM - 当我们需要使用外封胶涂抹LED产品时，应确保外封胶与LED封胶胶水相匹配，因为大多数LED的封装胶水为硅胶，它有较强的的氧化性和较强的吸湿性. When we need to use seal glue smearing LED products should be really warranty seal glue and led sealing glue glue to match, because most of the LED package glue is silica gel, it has strong oxidation resistance and strong hygroscopicity - 在开包装之前，请先检查包装袋有无漏气，如有漏气现象，请退回我司重新除湿包装再使用。 Before opening the package, please check the packing bag is free of leakage, if there is leakage phenomenon, please return the division to re - use the packaging and re - use - 要在规定环境条件中使用：温度30℃以内，湿度60%RH以下 To be used in the provisions of the environmental conditions - 折包使用前：请用70℃除湿24小时后才可以使用 Please use 70 degrees 24 hours before you can use the10 after the use of the material is more than 24H of the use of time is not used up - 材料拆装后使用时间超过24H未用完，需烘烤70℃/24小时除湿才可使用。 Need to bake 70 degrees /24 hours before the use of the desiccant can be used.