Approval Sheet Customer： Item： TOP VIEW LEDS Part No.: WLUB5050A-351-Y Customer P/N: 稳亮电子有限公司 WEN LIANG OPTO-ECTRONICS CO.LTD. Add: Building 6,Xiaweishui Industrial District, Hongqiaotou Songgang Street, Bao'an, Shenzhen, Guangdong, China (513000) PREPARED BY; CHECKED BY; APPROVED BY; SIGNATURE; CHECKED BY; APPROVED BY. 供方签章： 客户签章： SUOOLIER： CUSTOMER： 日期： 日期： DATE： DATE： 备注：承认签章后请回复一份（或复印件）给我公司，其余由贵司留作存盘。如果在签章的承认书（或复印件）回复我司之前，下了有关此零件的订单且又无特殊说明，那么我司就确定贵司已完全承认。 Please return one specification or one copy of it with your chop and signature of approval and retain the others for your record.In the event of an order being placed for this part number before the chop and signed with specification (or copy) is returnde and without special explanation,it will be assumed that full approval have been given. # TYPE NO: WLUB5050A-351-Y # PACKAGE DIMENSIONS Note: 1. All Dimensions are in millimeters 2. Tolerance is ±0.1mm Unless otherwise specified. Device Selection Guide Emitting Color; Lens Type. Blue; Water Clear. Absolute Maximum Ratings at Ta=25℃ Parameter; Symbol; Max; Unit. Power Dissipation; Pd; 0.2; W. Peak Forward Current 1/10 Duty Cycle,0.1ms Pulse Width; IFP; 150; mA. Forward Current; IF; 90; mA. Reverse Voltage; VR; 5; V. Operating Temperature Range; Topr; -40℃ to+100℃. Storage Temperature Range; Tstg; -40℃ to+110℃. Lead Soldering Temperature 【3mm From Body】; Tsol; 260℃ For 5 Seconds. Electrical Optical Characteristics at Ta=25℃ Parameter; Symbol; Min; Typ; Max; Unit; Condition. Forward Voltage; VF; 2.8; 3.2; 3.6; V; IF=3*20mA. Luminous Intensity; IV; 500; 1000; ---; mcd; IF=3*20mA. Dominant Wavelength; λd; 465; 470; 475; nm; IF=3*20mA. Luminous flux; ф; 3; ---; 4; lm; IF=3*20mA A. Reverse Current; IR; /; /; 5; μA; VR=5V. Viewing Angle; 2θ1/2; /; 120; /; deg; IF=60mA. Note. 1.Tolerance of Luminous Intensity:11% 2.Tolerance of Dominant Wavelength:+0.5nm 3.Toleance of Forward Voltage:+0.1V Typical Electro-Optical Characteristics Curves Soldering condition Reflow Soldering; Hand Soldering. Lead Solder; Lead – free Solder. Pre-heat; 120~150℃; 180~200℃; Temperature; 350℃ Max.. Pre-heat time; 120sec. Max.; 120sec. Max.; Soldering time; 3sec. Max. (one time only). Peak temperature; 240℃ Max.; 260℃ Max.. Soldering time; 10sec. Max.; 10sec. Max.. Condition; refer to Temperature- Profile 1; refer to Temperature- Profile 2. Temperature profile in re-flow machine for lead type and lead free type Storage condition In order to avoid the absorption of moisture, it is recommended to store in the dry box (or desiccators) with a desiccant. Otherwise, to store them in the following environment is recommended. Temperature: 5℃~30℃ Humidity: 60%HR max. Dimension of carrier tape Dimension of reel Notes: 1. All dimensions are in mm, tolerance is±2.0mm unless otherwise noted. 2. Specifications are subject to change without notice. Packing 1. Number of SMD 5050 product in reel and bag(anti-static bag) 1.1. 1000 piece of SMD 5050 Products on one reel. The reel will be put in the anti-static bag then sealed. 1.2. 1000 piece of SMD 5050 products on one bag.