Approval Sheet Customer： Item： 2*3*4 LED LAMP Part No.: WLUY612V63-19 Customer P/N: Customer Approval 稳亮电子有限公司 WEN LIANG OPTO-ECTRONICS CO.LTD. Add: 4/F, Building 6,Tian Fu An Industrial Zone,Jiu wei Village,xi xiang Town,bao an District shen zhen 518126,china Tel: 0086-755-29785289 Fax: 0086-755-29785189 Email: szwenliang@163.com Website: http://www.szwenliang.net.cn

|  | 2*3*4mm SQUARE LED LAMP |
| --- | --- |
|  | ## WLUY612V63-19 |
| # PACKAGE DIMENSIONS Note: 1. All Dimensions are in millimeters 2.Tolerance is ±0.25mm(0.010”) Unless otherwise specified. |  |
| # REV:A/0 DATE:2009-07-13 Page:1/4 |  |

|  | 2*3*4mm SQUARE LED LAMP |
| --- | --- |
|  | ## WLUY612V63-19 |
| Emitting Color; Lens Type. Yellow; Water Clear. Absolute Maximum Ratings at Ta=25℃ Parameter; Symbol; Max; Unit. Power Dissipation; PD; 80; mW. Peak Forward Current 1/10 Duty Cycle,0.1ms Pulse Width; IFP; 60; mA. Continuous Forward Current; IF; 30; mA. Derating Linear From 50℃; VR; 0.4; mA/℃. Reverse Voltage; VR; 5; V. Operating Temperature Range; Topr; -40℃ to+80℃. Storage Temperature Range; Tstg; -25℃ to+100℃. Lead Soldering Temperature 【4mm From Body】; Tsol; 260℃ For 5 Seconds. Electrical Optical Characteristics at Ta=25℃ Parameter; Symbol; Min; Typ; Max; Unit; Condition. Forward Voltage; VF; 1.9; ---; 2.4; V; If=20mA. Luminous Intensity; IV; 400; ---; 800; mcd; If=20mA. Dominant Wavelength; λd; 585; 590; 595; nm; If=20mA. Spectral Line Half-Width; △λ; ---; 30; ---; nm; If=20mA. Reverse Current; IR; ---; ---; 10; μA; VR=5V. Viewing Angle; 2θ1/2; ---; 90; ---; deg; If=20mA. |
| REV:A/0 DATE: 2009-07-13 Page:2/4 |  |

|  | 2*3*4mm SQUARE LED LAMP |
| --- | --- |
|  | ## WLUY612V63-19 |
| LED LAMP ELECTRONIC/OPTICAL CHARACTERISTICS CURVE |  |
| # REV:A/0 DATE: 2009-07-13 Page:3/4 |  |

|  | 2*3*4mm SQUARE LED LAMP |
| --- | --- |
|  | ## WLUY612V63-19 |
| Reliability test items and conditions : Criteria for Judging the Damage： Measuring Item; Symbol; Measuring Conditions; Judgement criteria for failure. Forward Voltage; VF; IF=20mA; OVER V* 120% OR 80%. Reverse Current; IR; VR=5V; OVER H*2. Luminous Intensity; IV; IF=20mA; L*0.5. Dominant wavelength; λD; nm; OVER±1.5nm(W). Note: - V and H means the upper limit of specified characteristics. L and W means initial value - Measurement shall be taken between 2 hours and after the test pieces have been returned to normal Ambient conditions after completion of each test. NO; Item; Test Conditions; Test Hours/Cycle; Sample Size; Judgment. 1; Solder Heat; TEMP：260±5℃; 5 SEC; 76 PCS; OK. 2; Temperature Cycle; H：+85℃ 30min ∫ 5min L：-55℃ 30min; 50 CYCLES; 76 PCS; OK. 3; Thermal Shock; H：+100℃ 5min ∫ 10set L：-10℃ 5min; 50 CYCLES; 76 PCS; OK. 4; High Temperature Storage; TEMP：100℃; 1000 HRS; 76 PCS; OK. 5; Low Temperature Storage; TEMP：-55℃; 1000 HRS; 76 PCS; OK. 6; DC Operating Life; TEMP：25℃; 1000 HRS; 76 PCS; OK. 7; High Temperature / High Humidity; 85℃ / 85%RH; 1000 HRS; 76 PCS; OK. |
| # REV:A/0 DATE:2009-07-13 Page:4/4 |  |